SM 136E

Software Application Manual		    # 136

QFLW-DLL      V32F

DLL Libraries 16 / 32 bit

for C-842 DC Motor Controllers

covers:

SID 045 : QFLW155.DLL (16 bit)

SID 062 : QFLW32F.DLL (32 bit)

SID#			136

Name:		QFLW-DLL package

Release:		3.0

Release Date:	02 October 1998

©  Physik Instrumente GmbH, 1998

Table of Contents :

DLL Libraries for C-842	3

DLL Function Reference :	7

void Set_BaseAddress(unsigned short address)	7

BOOL Board_Installed()	7

char Axis_Installed()	7

int InitBoard(char mode)	7

void Init_LS (char axis1, char axis2, char axis3, char axis4)	8

int autodetect()	8

void InitAxis(char axis)	8

void MoveA(char axis, long position)	9

void MoveA_12(long pos_1, long pos_2)	9

void MoveA_123(long pos_1, long pos_2, long pos_3)	10

void MoveR(char axis, long counts)	10

void MoveR_12(long pos_1, long pos_2)	10

void MoveR_123(long pos_1, long pos_2, long pos_3)	10

void VectorA_12(long v_path, long pos_1, long pos_2)	10

void VectorR_12(long v_path, long shift_1, long shift_2)	11

long WaitStop(char axis)	11

BOOL moving(char axis)	11

long get_pos(char axis)	12

long get_PosErr(char axis)	12

unsigned short get_stat(char axis)	12

void get_pos4(TQ_long *Pos4)	12

void execute(char axis,Tcmd command,long parameter, char *report)	12

void translate(char *command, char *report)	13

void axis_select (char axis)	13

void setQMC(char hexcommand, long parameter)	13

long getQMC(char hexcommand)	13

Example: Using QFLW(16bit) DLL in a C Program:	15

# DLL Libraries for C-842

The functionality of the C-842 motor controller is available for windows programmers by Dynamic Link Libraries offering a variety of functions for board handling and motion control.

Two libraries with the same functionality are available:

16 bit:	QFLW155.DLL
32 bit:	QFLW32D.DLL

The libraries can be used with all programming environments like Visual-C++, Delphi1.0/2.0, MS Visual Basic.

Frequently used commands like Move Absolute, Move Relative or position queries like Tell Position can be called by library functions. Other commads like Set P-gain or Tell following error can be accessed via the TRANSLATE and/or EXECUTE function. The actual command then is passed as a parameter to the library function.

More than 100 commands are defined as constants to be accessed with the EXECUTE function. Packaging the command in a string in conjunction with axis identifier and parameter is the other way to send a command via the TRANSLATE function.

Examples:	execute(3,MR,45000,report);

translate("3MR45000",report);

Both functions perform the same action, moving motor#3 relative for 45000 counts.

In the same manner, the complete QMove commands set of more than 100 commands can be called.

The command transfered is executed immediately when received.

For detailed description on the commands implemented refer to 
Programming Manuals MS45E and MS46E.

Functions are exported by name and ordinal number:

get_pos	index 2
	get_PosErr	index 4
	get_stat	index 6
	get_pos4	index 8
	execute	index 10
	translate	index 12
	moving	index 14
	Set_BaseAddress	index 16
	Axis_Installed	index 18
	InitBoard	index 20
	InitAxis	index 22
	Board_Installed	index 24
	MoveA	index 26
	MoveR	index 28
	WaitStop	index 30
	Init_LS	index 32
	StatusBoard	index 34
	axis_select	index 36
	setQMC	index 38
	getQMC	index 40
	autodetect	index 42
	set_pos	index 44
	MoveA_12	index 50
	MoveA_123	index 51
	MoveR_12	index 52
	MoveR_123	index 53
	VectorA_12	index 60
	VectorR_12	index 61

Declarations of functions: (Pascal style)

Note: All functions of QFLW21D.DLL are also available in QFLW32F.DLL.

function  get_pos(axis:byte):longint; external'QFLW32D.DLL';

function  get_PosErr(axis:byte):longint;  external'QFLW32D.DLL';

function  get_stat(axis:byte):integer; external'QFLW32D.DLL';

procedure get_pos4(var pos4:TQ_long); external'QFLW32D.DLL';

procedure execute(axis:byte;command:TCmd;para:longint;var report:PChar);

external'QFLW32D.DLL';

procedure translate(cmd:Pchar; rep:Pchar); external'QFLW32D.DLL';

function  moving(axis:byte):boolean; external'QFLW32D.DLL';

procedure Set_BaseAddress(address:word); external'QFLW32D.DLL';

function  Axis_Installed:byte; external'QFLW32D.DLL';

function  InitBoard(mode:byte):integer; external'QFLW32D.DLL';

procedure InitAxis(axis:byte); external'QFLW32D.DLL';

function  Board_Installed:boolean;external'QFLW32D.DLL';

procedure MoveA(axis:byte; position:longint); external'QFLW32D.DLL';

procedure MoveA_12(pos1:longint; pos2:longint); stdcall external'QFLW32F.DLL';

procedure MoveA_123(pos1:longint; pos2:longint; pos3:longint); stdcall external'QFLW32F.DLL';

procedure MoveR(axis:byte; counts:longint), external'QFLW32D.DLL';

procedure MoveR_12(shift1:longint; shift2:longint); stdcall external'QFLW32F.DLL';

procedure MoveR_123(shift1:longint; shift2:longint; shift3:longint); stdcall external'QFLW32F.DLL';

procedure VectorA_12(v_path, pos1, pos2:longint); stdcall external'QFLW32F.DLL';

procedure VectorR_12(v_path, shift1, shift2:longint); stdcall external'QFLW32F.DLL';

function  WaitStop(axis:byte):integer; external'QFLW32D.DLL';

procedure Init_LS(drive1,drive2,drive3,drive4:byte);external'QFLW32D.DLL';

function  StatusBoard:word; external'QFLW32D.DLL';

procedure axis_select(axis:byte); external'QFLW32D.DLL';

procedure setQMC(cmd:byte; para:longint); external'QFLW32D.DLL';

function  getQMC(cmd:byte):longint; external'QFLW32D.DLL';

function  autodetect:integer; external'QFLW32D.DLL';

procedure set_pos(axis:byte; newpos:longint); external'QFLW32F.DLL';

Declarations of functions: (C style)

long get_pos(char axis);

long get_PosErr(char axis);

int get_stat(char axis);

void get_pos4(TQ_long *pos4);

void execute(char axis,TCmd command,long para,;char *report);

void translate(char *cmd, char *rep);

BOOL moving(char axis);

void Set_BaseAddress(unsigned short address);

char Axis_Installed();

int InitBoard(char mode);

void InitAxis(char axis);

BOOL Board_Installed();

void MoveA(char axis, long position);

void MoveA_12(long pos1, long pos_2);

void MoveA_123(long pos1, long pos_2, long pos_3);

void VectorA_12(long v_path, long pos_1, long pos_2);

void MoveR(char axis, long counts);

void MoveR_12(long shift_1, long shift_2);

void MoveR_123(long shift_1, long shift_2, long shift_3);

void VectorR_12(long v_path, long shift_1, long shift_2);

int WaitStop(char axis);

void Init_LS(char drive1,char drive2,char drive3,char drive4);

unsigned short StatusBoard();

void axis_select(char axis);

void setQMC(char cmd,long para);

long getQMC(char cmd);

int autodetect();

void set_pos(char axis, long newpos);

Types Constants and Structures

Types, constants and functions are defined in definition text files that can be used directly by Pascal and Delphi programmers:

QFLW32D.DEF	for 32 bit systems
QFLW155.DEF	for 16 bit systems

Also the definition files includes the definition of the C842 commands that can be sent via the execute function.

Types used by the QFL library:

Tstr = string[5];

Tcmd = record

code   : byte;

qclass : byte;

lab    : Tstr;

end;

TQ_long    = array[1..4] of longint;

TQ_integer = array[1..4] of integer;

All commands (cmd) used with the execute procedure are defines as constants of the type TCmd. Names of the variables comply with command names used in the QMove program and are documented in the MS45E operating manual.

Example of a command constanf of TCmd type, here SA(Set Acceleration)

SA : Tcmd = (code:$12; qclass:wr_U1616;   lab:'SA:');  (* SET_ACC        *)

In the same manner all (more than 100) short form commands are defined. For further information see the definition file QFLW32D.DEF.

General structure of a program using the QFLW library:

Define board address

Detects max axis number and adjusts sample rate to 400 µs

Sets the board function register

Reads and defines the limit switch configuration

Sets Motor #1 to default parameters

Sets Motor #2 to default parameters

Move Command (main program)

main program

Set_BaseAddress(....)

MaxAxis := Axis_Installed

status := InitBoard(9)

status := autodetect

InitAxis(1)

InitAxis(2)

MoveA(1,5000)

translate('1SV80000',report)

...

...

end.

## DLL Function Reference :

System Initializing Functions

### void Set_BaseAddress(unsigned short address)

This procedure allows to define the hardware I/O addrees selcted by the DIP switches on the C-842 board. Default setting is 0x210.

If the C-842 board is used with the default address setting (0x210), this function is not required.

Ordinal index:	16

Input:			address as unsigned short (word, 16 bit)

Return:		none

### BOOL Board_Installed()

This function returns true or false if a board is installed at the default address or the address specified with the set_BaseAddress function.

Ordinal index:	24

Input:			none

Return:		boolean value (0 or 1)

### char Axis_Installed()

This function returns the number of motor axes supported by the processor. There are 2-axes and 4-axes version available (C-842.20 and C-842.40).

Ordinal index:	18

Input :		none

Return:		number of axes as char (byte)

### int InitBoard(char mode)

This function initializes the C-842 and returns the board status. It has to be sent after the C-842 is powered up to enable the amplifiers and to define the encoder modes. Mode is the byte (unsigned char) transferred to the board function register.

Following actions are performed:

- Write mode to board function register

- Board reset

- Set output to DAC16

- Set Limit Sense to $00FF

The normal init mode value is 9.

Ordinal index:	20

Input:			mode as char (byte)

Return:		status as int

### void Init_LS (char axis1, char axis2, char axis3, char axis4)

This procedure sets the Limit_Sense Register Byte.

After power up the System the limit switch wiring and levels have to be defined.

If no limit switches are connnected, the axis parameter has to be set to 0 for the corresponding axis.

If limit switches are wired compatible with PI standard stages, the corresponding Byte(char) has to be set to 1.

Example: One PI-compatible stage is connected to axis #2. All other axes are not connected or have motors without limit switch connection, call init_LS with these parameters:  Init_LS(0,1,0,0)

Note: Init_LS() overwrites autodetect settings if called afterwards.

Ordinal index:	32

Input:			mode as byte (char)

Return:		status as integer (int)

### int autodetect()

This function reads the current limit switch lines levels of all axes and defines the reading as normal state (no switch tripped). Any deviation of the value read is interpreted as a limit switch hit event, triggered by the stage.

Note: When calling autodetect, no limit switch must be hit.
	Note: autodetect replaces prior Init_LS procedures.

autodetect performs these functions:

- reads the current limit switch state

- analyses whether one limit switch is hit and returning error code in that case

- sets the limit_sense state of the board according to the values found

- sets LimitMode variable to the limit_sense value

Ordinal index:	42

Input:	none

Return:	value of detected LS configuration (e.g. 255 if no LS connected)
	-1 if error (LS was hit)

### void InitAxis(char axis)

This procedure initializes the specified axis and sets default values. The settings may not match your specific application, so it might be required to set individual servo parameters, velocities and accelerations.

InitAxis performs these functions: SET_KP, Set_VEL, Set_ACC, AUTO_STOP_OFF, Upate_Filter, CLR_STATUS

Ordinal index:	22

Input:	axis as char (byte)

Return:	none

Moving Functions

### void MoveA(char axis, long position)

Move Absolute: This function starts one or all motors to move to an absolute position. If the axis identifier is 0, all axes available are moved to the same position.

After the function is called, the motion is started and the function is left immediately. If the next call of MoveA occurs before the motor(s) have terminated their moves, the old target is replaced by the new one and the motors immediately head towards their new targets, even if the previous target(s) were not yet reached. In order to synchronize motions and to start the next movement not before the first target is reached, the WaitStop command has to be called before the next move command is executed.

Example 1 :At first, motor#1 will run to position 5000 and stop there, then run to zero position:

MoveA(1,5000)
WaitStop(1)
MoveA(1,0)

Example 2:	Assume the motor is at zero position. Without the WaitStop command the first move to 85000 will not be performed and the motor will already stop at position 1000:

MoveA(1,85000)
MoveA(1,1000)

Ordinal index:	26

Input:	axis : byte, position : long integer

Return:	none

### void MoveA_12(long pos_1, long pos_2)

Move Absolute XY with synchron start. This function starts axis 1 and 2 with individual velocities at the same time (synchron start).

After the function is called, the 2-axis motion is started and the function is left immediately. The general behavior is similar like the MoveA function.

Example : Set velocities of axes 1 and 2 and start simultaneous motion:

translate("1SV20000",report)
translate("2SV80000",report)
MoveA_12(25000,75000)

Ordinal index:	50

Input:	position1 as long, position 2 as long

Return:	none

### void MoveA_123(long pos_1, long pos_2, long pos_3)

see MoveA_12()

### void MoveR(char axis, long counts)

Move Relative: This function starts a motor for a relative run. The specified count number is added to the current position and the result is stored as the new target.

After the function called, the motion is started and the function is left immediately. If the next call of MoveR occurs before the motor(s) have terminated their moves, a continuous motion to the new target is performed. In order to synchronize motions and to start the next movement not before the last move is terminated, the WaitStop command has to be used.

If the axis identifier is 0, all axis available are moved by the same number of counts.

Ordinal index:	28

Input:	axis as char(byte), counts as long

Return:	none

### void MoveR_12(long pos_1, long pos_2)

Move Relative XY with synchron start. This function starts axis 1 and 2 with individual velocities at the same time (synchron start).

After the function is called, the 2-axis motion is started and the function is left immediately. The general behavior is similar like the MoveR function.

Example : Set velocities of axes 1 and 2 and start simultaneous motion:

translate("1SV20000",report)
translate("2SV80000",report)
MoveR_12(25000,75000)

Ordinal index:	52

Input:	position1 as long, position 2 as long

Return:	none

### void MoveR_123(long pos_1, long pos_2, long pos_3)

see MoveR_12()

### void VectorA_12(long v_path, long pos_1, long pos_2)

Move axes 1 and 2 with given path velocity to coordinates points X/Y.

The function starts both axes at the same time with that velocity, that the resulting path velocity is constant and both axes arrive the target at the same time.

Example : move from the current position to the absolute point 30000/98000 with the path velocity of 6500 counts/s:

VectorA_12(6500,30000,98000)

Ordinal index:	60

Input:	path velocity as long, coordinates x and y as long

Return:	none

### void VectorR_12(long v_path, long shift_1, long shift_2)

Move axes 1 and 2 with given path velocity relative to the current positon by shift_1 and shift_2 counts.

The function starts both axes at the same time with that velocity, that the resulting path velocity is constant and both axes arrive the new target at the same time.

Example : move from the current position relative x by 30000 and Y by 60000 counts at the path velocity of 7000 c/s:

VectorA_12(7000,30000,60000)

Ordinal index:	61

Input:	path velocity as long, coordinates x and y as long

Return:	none

### long WaitStop(char axis)

The WaitStop function controls the dynamic position of the specified axis and returns if the motor has reached the target. It can be used to synchronize multiple movements.

Ordinal index:	30

Parameter:	axis as char (byte)

Return:	none

### BOOL moving(char axis)

This function returns true if the trajectory of the specified axis has not yet reached the target position. The function reads the on-target flag of the condition register.

Ordinal index:	14

Parameter:	axis as char (byte)

Return:	boolean

Reading Functions

### long get_pos(char axis)

This function reads the current position of the indicated motor.

Ordinal index:	2

Parameter:	axis as char (byte)

Return:	current encoder position as long

### long get_PosErr(char axis)

This function returns the current deviation of the encoder position from the profile position in encoder counts.

Ordinal index:	4

Parameter:	axis as char (byte)

Return:	error counts as long

### unsigned short get_stat(char axis)

This function returns the status word of the indicated axis.

Ordinal index:	6

Input:	axis as byte

Return:	status value as unsigned short (word)

### void get_pos4(TQ_long *Pos4)

This function reads the current posiltions of all 4 axes and writes it into an array of 4 position.

Ordinal index:	8

Input :	array[1..4] of long as TQ_long

Return:	none

### void execute(char axis,Tcmd command,long parameter, char *report)

This function calls a processor function. Equivalent to translate procedure.

Ordinal index:	10

Input:	axis as char (byte)
command as Tcmd
parameter as long
report as string in pascal convention. Defined as an array[0..255] of char, where the first char report[0] is the length of the string. The string actually starts at report[1].

Return:	none

### void translate(char *command, char *report)

Ordinal index:	12

Parameters:	command as zero terminated string, format is aCn, with
a:   motor axis identifier (0, 1 or 2), if it is zero, then the command is applied to both axes, otherwise only to the specified axis.

C:   Command, see command list,
n:   parameter value as string

report: 	pointer to C string, supplied by the calling function.
The calling program should define a character array of 256 characters to take the report message.

Example: translate(´2MA5400´, report);

Low Level Functions

### void axis_select (char axis)

This function activates the specified axis and updates an internal axis flag. Normal move commands do not need the axis_select function, except setQMC or getQMC are used.

Ordinal Index:	36
Input:	axis as char (byte)
Return:	none

### void setQMC(char hexcommand, long parameter)

This function writes a genuine command and the related parameter to the activated axis. Use axis_select function to choose one axis prior to call setQMC. axis_select is only required if the axis should be changed.

Ordinal index:	38
Input:	hexcommand as char (byte)
Return:	none

### long getQMC(char hexcommand)

This function returns the questioned value. as long as the query does not concern a global command, the value for the activated axis is returned. Use axis_select function to choose one axis prior to call getQMC. axis_select is only required if the axis should be changed.

Ordinal index:	40
Input : 	command in hexcode as char (byte)
Return:	value as long integer

Special Application :

ClearAxisStatus

In order to clear the status word of one axis, the follwing sequence should be used:

axis_select (axisnumber)
	setQMC (0x33,0)

## Example: Using QFLW(16bit) DLL in a C Program:

////////////////////////////////////////////////////////////////////////

// C, C++ and VC++ Software Example for C-842 Motor control

// Copyright (C) 1996 PHYSIK INSTRUMENTE GmbH

// All rights reserved.

///////////////////////////////////////////////////////////////////////

// This C source fragment is just an example and has to be

// modified according to your development system.

////////////////////////////////////////////////////////////////////////

; io842.def

; Definition file declaring module parameters for the application.

NAME         IO842

DESCRIPTION  'IO842 Windows Application'

EXETYPE      WINDOWS

CODE         PRELOAD MOVEABLE DISCARDABLE

DATA         PRELOAD MOVEABLE MULTIPLE

HEAPSIZE     1024   ; initial heap size

; Stack size is passed as argument to linker's /STACK option

IMPORTS QFLW155.Set_BaseAddress

QFLW155.Board_Installed

QFLW155.Axis_Installed

QFLW155.InitBoard

QFLW155.Init_LS

QFLW155.autodetect

QFLW155.InitAxis

QFLW155.MoveA

QFLW155.MoveR

QFLW155.moving

QFLW155.get_pos

QFLW155.translate

///////////////////////////////////////////////////////////////////

// io842.h		header file

/////////////////////////////////////////////////////////////////////////////

/* using qflw155.dll */

/* ****** declaring QFLW155 functions ******* */

/* System initializing functions */

#ifdef _cplusplus

EXTERN "C" {

#endif

void FAR PASCAL Set_BaseAddress( WORD wAddress );

BOOL FAR PASCAL Board_Installed();

BYTE FAR PASCAL Axis_Installed();

int  FAR PASCAL InitBoard( BYTE mode );

void FAR PASCAL Init_LS( BYTE axis1, BYTE axis2, BYTE axis3, BYTE axis4 );

int  FAR PASCAL autodetect();

void FAR PASCAL InitAxis( BYTE nAxisNo );

/* Moving functions */

void FAR PASCAL MoveA( BYTE nAxisNo, long position );

void FAR PASCAL MoveR( BYTE nAxisNo, long counts );

BOOL FAR PASCAL moving( BYTE nAxisNo );

/* query functions */

long FAR PASCAL get_pos( BYTE nAxisNo );

/* generic functions */

void FAR PASCAL translate( char* strCommand, char strReport[64] );

char strReport[64] );

#ifdef _cplusplus

}

#endif

///////////////////////////////////////////////////////////////////

// io842.cpp : Defines the class behaviors for the application.

///////////////////////////////////////////////////////////////////

#define _cplusplus

#include "io842.h"

///////////////////////////////////////////////////////////////////

// CIo842App initialization

BOOL Initialize_Board()

{

int nResultLimit;

// check if board present

if( !Board_Installed() )

return FALSE;

InitBoard(9);

BYTE nAxisPresent = Axis_Installed();

BYTE i;

// call the setup function to analys and set the limit switches

nResultLimit = autodetect();

if( nResultLimit == -1 )

MessageBox( NULL, "Autodetect() can not detect Limit Switch state.\nLimit Switches are disabled!\0", "New Building Demo\0", MB_OK | MB_ICONSTOP );

// initialize all axis with default parameters

for( i=1; i<nAxisPresent+1; i++ )

InitAxis(i);

return TRUE;

}

void CIo842App::Init_C842_forCycles()

{

BYTE  nAxisNo;

char strReport[128];

BOOL bLimitReached;

int n100mSecs = 0;

// set motion parameters :

translate( "DP120\0", strReport );

translate( "SA90\0", strReport );

translate( "DI5\0", strReport );

translate( "DL1200\0", strReport );

translate( "1SV7000\0", strReport );

translate( "1SV30000\0", strReport );

// move motors

translate( "1MR50000\0", strReport );

translate( "2MR150000\0", strReport );

while( (moving(1) || moving(2))  && (n100mSecs < 100) )

{

Delay(100);

n100mSecs++;

}

// combine movements :

for( nAxisNo=1; nAxisNo<3; nAxisNo++ )

{

MoveR( nAxisNo, -1000 );

}

}