Uncertainty Calculations

Reference Guide

| Function | Uncertainty | Example (a = 41.2 ± 1.6, b = 3.02 ± 0.43) |  |
| --- | --- | --- | --- |
|  |  | a+b a−b | 44.2 ± 1.7 38.2 ± 1.7 |
| Δzz=Δxx2+Δyy z=x/y | Δzz=Δxx2+Δyy2 | a×b a/b | 124 ± 18 13.6 ± 2.0 |
| z = xn | Δzz=nΔxx | b3 a | 28 ± 12 6.42 ± 0.12 |
| z = ln x | Δz=Δxx * | ln a | 3.72 ± 0.04 |
| z = f(x) | Δz=dfdxΔx | sin a° = sin(0.719±0.028 rad) | 0.659 ± 0.021 (0.021 = 0.028 cos a°) |
| z = f(x) | Δz=12fx+Δx−fx−Δx | sin a° log10 b | 0.659 ± 0.021 0.48 ± 0.06 |

Notes

- Quote uncertainty to no more than 2 significant figures.  For final results, most people would quote 2 significant figures if the first figure is 1 or 2, and 1 sig. fig. otherwise: 4.324 ± 0.012, but 4.32 ± 0.04.  For intermediate results, it is reasonable to keep 2 sig. fig. to guard against rounding errors.

- Quote the value to the same number of decimal places as the uncertainty: 4.324 ± 0.012, 4.32 ± 0.04, 4.3 ± 1.2.  If the number is to be expressed in scientific notation, use a common exponent: (4.324 ± 0.012) × 108, not 4.324 × 108 ± 1.2 × 106.  Confusion may arise if there is a trailing zero (is ±20 one significant figure or two?); scientific notation helps in this case (±0.2 × 102 is definitely one sig. fig.; ±0.20 × 102 is definitely two).

- The “brute force” method (last row of table) always works: if in doubt, do it that way.

- What to do if z = f(x,y), i.e. a function of two variables:

  - Calculate the uncertainty in z caused by the uncertainties in x and y separately:
	Δzx=12fx+Δx,y−fx−Δx,y
	Δzy=12fx,y+Δy−fx,y−Δy.

  - Take the square root of the sum of squares:
	Δz=Δzx2+Δzy2.

- It is possible to quote asymmetric errors, e.g. 2.2±0.7−1=0.45−0.11+0.21.  This should only be necessary if the fractional error is large (30% in this case).  It makes further manipulation of the uncertainties awkward, so don’t do it unless the upper and lower error bars are significantly different.

* Note: this formula does not apply to z = log10 x!